BUG: 8057 – Fehlerhafte Rüstzeit bei Reduzierung durch Teilrückmeldung

| Release: | 7.40 |
| --- | --- |
| Class: | /GIB/CL_DCP_MAIN_UTILITIES |
| Method: | GET_OPERATION_TIMES |
| *#Setup Time --------------------------------------------------------------------------* *#Porccess orders do not have a actual Setup Time so this must be read from the *#operation defaults if the Default field for Setup Time is specified in DCP Area Settings *#if the Default field is not specified in the DCP Area Settings then just take the normal *#Setup Time field from KBEDD *#HW8057 - Fehlerhafte Rüstzeit (SLI 27.09.2017 13:26:56) * IF ir_settings->ms_dcp_time-setup_adj_field IS NOT INITIAL. IF ir_settings->ms_dcp_time-setup_adj_field IS NOT INITIAL AND ir_operation->mr_order->ms_caufvd-autyp = '40'. *##HW8057 - Fehlerhafte Rüstzeit (SLI 27.09.2017 13:26:56) CONCATENATE 'VGW0' ir_settings->ms_dcp_time-setup_adj_field INTO lf_default. CONCATENATE 'VGE0' ir_settings->ms_dcp_time-setup_adj_field INTO lf_default_unit. ASSIGN COMPONENT lf_default OF STRUCTURE ls_afvgd TO <lf_default>. ASSIGN COMPONENT lf_default_unit OF STRUCTURE ls_afvgd TO <lf_default_unit>. /gib/cl_dcp_order_services=>conversion_time( EXPORTING if_value_in = <lf_default> if_unit_in = <lf_default_unit> CHANGING ef_unit_out = lf_second ef_value_out = ef_setup ). ELSE. /gib/cl_dcp_order_services=>conversion_time( EXPORTING if_value_in = ls_kbedd-kruerest if_unit_in = ls_kbedd-keinh CHANGING ef_unit_out = lf_second ef_value_out = ef_setup ). ENDIF. |  |

| Release: | 7.30 |
| --- | --- |
| Class: | /GIB/CL_DCP_BASICS |
| Method: | DURATION_IN_SECONDS |
| if_value_in = ls_kbed-kruerest if_unit_in = ls_kbed-keinh CHANGING ef_unit_out = lf_second ef_value_out = lf_ruest ). *#if the Setup time is initial and the SETUP FIELD is specified *#in the area settings read the setup time from the operation IF lf_ruest IS INITIAL AND *#HW8057 - Fehlerhafte Rüstzeit (SLI 27.09.2017 13:28:59) ms_header-autyp = '40' AND"only for proccess orders *##HW8057 - Fehlerhafte Rüstzeit (SLI 27.09.2017 13:28:59) lr_settings->ms_dcp_time-setup_adj_field IS NOT INITIAL. CONCATENATE 'VGW0' lr_settings->ms_dcp_time-setup_adj_field INTO lf_default. CONCATENATE 'VGE0' lr_settings->ms_dcp_time-setup_adj_field INTO lf_default_unit. ASSIGN COMPONENT lf_default OF STRUCTURE ls_afvgd TO <lf_default>. ASSIGN COMPONENT lf_default_unit OF STRUCTURE ls_afvgd TO <lf_default_unit>. /gib/cl_dcp_order_services=>conversion_time( EXPORTING if_value_in = <lf_default> if_unit_in = <lf_default_unit> CHANGING ef_unit_out = lf_second ef_value_out = lf_ruest ). ENDIF. *##HW7490 - Rüstzeiten verschwinden bei manueller Rüstzeitanpassung (MPL 10.05.2017) *#Processing Time -------------------------------------------------------------------* /gib/cl_dcp_order_services=>conversion_time( EXPORTING if_value_in = ls_kbed-kbearest |  |